Schneider M580 Sorting Line – FDS

Confidential

Erik van der Wal

Table of contents

Abbreviations	4

General	5

Applications	5

EcoStruxure Control Expert Classic (Control Expert XL + M580 Safety - V16.0)	5

SoMove	5

EcoStruxure Automation Device Maintenance	5

Common Platform Issues	6

Virtual Machine	9

Setup – Serial	9

Setup – Ethernet NAT	13

Setup – USB-C Ethernet (Bridged)	14

Programming	15

PC  PLC	15

PLC  PC	17

Hardware	18

Importing ESD File	18

Updating DTM Catalog	21

Communication Module	23

Communication Network	25

Drive	26

Pre-requisites	26

Local Network Settings	27

Connection	28

Add In Project (Fixed IP)	29

Add In Project (DHCP by PLC)	32

Fieldbus Discovery	33

Setup	34

Parameters	35

Load configuration	37

FDR	40

M/S Mode	41

CIP	42

Firmware	47

Pre-requisites	47

Adding Device	48

Connecting to device	51

Preparing firmware	52

Updating firmware	53

Recovery device	56

OPC-UA	57

Server - Advanced	57

Server - Standard	62

Client (Testing)	64

RIO or LIO	66

Pre-requisites	66

Adding Location (LIO or RIO)	66

TeSys Island	73

Pre-requisites	73

Project configuration (Control Expert)	74

Adding Location (Motor group)	74

Name	77

Main Island Configuration	78

Add Avatar (Motor)	78

Settings Avatar (Motor)	80

Copy and Paste - Device	82

Setting up Island (Online)	84

# Abbreviations

This document uses several abbreviations, therefore the explanation for the abbreviations is listed below:

| Term | Description |
| --- | --- |
| CM | Control module |
| EM | Equipment module |
| UN | Unit |
| PC | Process Cell |
| fb | Feedback |
| CS | Control Signal |
| HMI | Human-Machine Interface |
| STS | Status |
| ALM | Alarm |
| PAR | Parameter |
| FP | Faceplate |
| En | Enable |
| WRN | Warning |
| Dly | Delay |
| Ti | Timer |
| INT | Integer |
| UDT | User Defined Type |
| AI | Analogue input |
| AO | Analogue output |
| LL | Low Low |
| L | Low |
| HH | High High |
| H | High |
| DOL | Direct Online |
| UI | User Interface |
| Repository | Often a name for a remote map, containing stored data |
| Baler | Bollegraaf machine, that can press materials into a bale |
| FIFO | First In, First Out Object that reports first, if the first to be handled |
| End-user | Can be an operator, or a Bollegraaf (service) engineer |

## External Links

| Document/File | Version | Usage |
| --- | --- | --- |
| M580 Programming - Guide | 02-05-2025 | Guide to program M580 platform and components |

# General

NOTE: 	Before setting up any Schneider – Order/Project, please read the M580 Programming Guide

## Overall

The overall program structure is setup in a way, to accomplish the following key aspects:

- Ease of use (FB’s have pre-installed Global variable links)

- Clear flow of data (Hardware signals: Inputs (program)  Process  Outputs)

- Flexibility of different divers/motor objects (each object can be a Motor starter, a drive or a simple relay)

- Clear variable naming (naming of variables should be clear)

- All in depth code = Structured Text, only overview block are FB (only intended adjustable area’s are FB based)

## Overview

Most of the program blocks are written or interconnected in such a way, that the following structure can be applied to most of the Objects:

At the current software version, this structure is currently in place for most objects and features. The basic structure consists of 3 main processes (independently executable processes) , an Input process, a main Process and an Output process. These processes are linked through global variables to exchange data between each other.

## Detailed

In the previous paragraph, an overview is provided of the program structure in a simplistic way. The figure below indicates the process structure with more details on very important data (like safety or others)

# Processes

# Functions

## Object

An Object instance is basically a single Motor instance, but bridging the functional aspect between machine/system and the motor. The Object instance can be set as a conveyor, a motor of a screen, a chain hoist of a bunker, external system Optical Sorter, etc.

### General

#### Limits

The Object has several limit functionalities in place to keep some crucial values in check. Limits are in place for;

| Variable | Function / purpose | Lower limit (min.) | Upper limit (max.) | Advised value |
| --- | --- | --- | --- | --- |
| SpeedRPM |  |  |  |  |
| JogRPM |  |  |  |  |
| PercentageRunning |  |  |  |  |
| FeedbackTimeStart |  |  |  |  |
| FeedbackTimeStop |  |  |  |  |
| FeedDelay |  |  |  |  |
| OFFDelay |  |  |  |  |

#### Factory Reset

### Interface

Object can be interfaced with by several means, to keep the object flexible to fur fill many roles.

### Manual

### Auto

### Pause

### Feed

### Motor (Driver)

### Offline

### Alarms

### Settings

## Motor

### General

### Control (Forward  Reverse

### Driver – VFD

### Driver – Tesys

### Driver – DOL

### Statistics

### Alarms

## Faction

### Manual

### Auto

### Runtime

### Statistics

### Offline (Group)

### Alarms

### Settings

## Storage

### General

#### Description

#### Parameters

#### Configuration

### Conveyor

#### Auto

#### Jog

### Doors

#### Auto Forward

#### Auto Reverse

#### Retry

#### No Sensor

#### Group

### Fill

### Alarms

### Settings

## Line Manager

## Storage Manager

## MCC

### Doors

### Climate Control

### Power Supply

### Transformer

### Indicator

### Claxon & Beacon

### Hour Counter

### Alarms

### Settings

## HMI

### Object

### Storage

### Block Indicator

### Recipes

### Emergency